A Novel Method: Application of CT based 3D model in operation with tablet device

Background: After the introduction of the smartphone in 2007, mobile devices such as tablets have become  important due to their unique features, and distinguished user experiences compared to prior products. Following this breakthrough, different kinds of mobile medical applications have been developed.

Prior to the mobile device experience, there were efforts to utilize 3D images in medical applications, however, these efforts proved futile because the available commercial products were expensive, and most were designed on huge platforms. Moreover, limitations remained in understanding real 3D body structures, because these 3D structures were based on 2D still images.

Purpose: This study reports our experience of using an ‘iPad’ to view, and rotate a 3D model by directly spinning a surgeon’s finger during an operation.

Methods: We made a 3D object of vessel, soft tissue, and bony structure from an abdominal CT scan using ‘3D-DOCTOR’, a PC based rendering program. To view the model on an iPad, the rendered 3D model in '.3ds' format, was transported into ‘3DVIA’, an iPad 3D viewer solution.

Results: The 3D object created by a PC based 3D reconstruction program worked well on tablet device (iPad).

Conclusion: Although in a test phase, with unresolved issues, the direct intra-operative handling of a patient tailored 3D model with a tablet device could be a useful tool compared to conventional still-cut 2D images.

Keywords: Tablet device, iPad, 3D, navigation surgery

Introduction

After Apple released the iPad in January 20101, many different kinds of devices have been introduced, and one such kind is ‘tablet device’ in the market.2

These new devices, though in similar appearance to previous products (notebook, netbook, and tablet PC),  they differ in technical characteristics. Unlike the previous devices that come with a desktop OS such as Windows or Mac OS X, a tablet device ships with a mobile operating system (OS). Additionally, these devices have touch sensitive screens to replace input devices such as keyboard, mouse, and stylus; have long battery life, have user friendly interface, and are lightweight

People attending in medical fields tried to apply the new device into clinical practice. Recently, several hospitals started using tablet devices; in assisting EMR (electrical medical record), referring to laboratory results and radiologic images, prescribing medications, and calculating the dosage of medication.3-5

Aside, ‘Navigation Surgery’, which is a well-known concept that uses a 3D reconstructed image from patient’s CT or MRI in surgery described in the literature,6 there are  expensive commercialized devices already used in this field.

In 2011, researchers at Kobe University, Japan, in studying the usability of mobile devices  used  iPad to aid Navigation surgery operations. Based on a patient CT scan, they made a 3D reconstructed image with ‘Osirix’, a 3D reconstructing program. And then they used this image with iPad at surgery. The related movie entitled as 'World 1st iPad Surgery: Osirix 3D navigation' could be found on YouTube (www.youtube.com).7 This was a simple study, but had an impact on further studies such as how to use iPad with a sterile plastic bag and surgical glove.8

However, studies mentioned above had limitation in using a 3D medical image on iPad. They only used 2 dimensional still pictures or movie clips instead of a real 3D object made on PC.

There are two benefits of  using a 3D object instead of a 2D picture image of a 3D object on iPad.

Firstly, an export of the 3D object created by a 3D medical program to a conventional standard file format means that a secondary processing is available. For instance, a 3D reconstructed CT object imported from the commercial 3D programs such as AutoCAD and 3D Studio Max could be further processed for educational purpose, used to create an actual model by 3D printer.

Secondly, the use of 2D image makes it impossible to approach  the right angle that a surgeon needs during an operation.. As the human body is a 3D  structure, there are planes and angles that visually overlapped during operation. Therefore, it is more useful to handle a 3D model directly than referring to only 2D images in operative field.

Results

This entire process from making a 3D object to using it on iPad was as follows. A 3D object was made based on CT images through a 3D reconstructing program in Windows Workstation, and was exported to a conventional 3D file format. The 3D object was then worked by a 3D viewing application on iPad as zooming, flipping, and rotating with fingers.

Of all the FDA (U.S Food and Drug administration) approved medical 3D programs, the Able software’s ‘3D-DOCTOR’ is the only program that supports external exporting of 3D object. In the step of 3D reconstructing, a 3D-DOCTOR makes a 3D object based on RAW DICOM images (CT, MRI, and PET CT) similar to other Medical 3D reconstructing programs.

A range based on HU (Housefield Unit) radio-density in one or several sections cut of a CT scan can be selected and applied to the whole sections. After that, a 3D object is built using a volume or surface rendering method. Note that ROI (Region of Interest) function could also be used for manual correction.

(Fig. 1. 3D reconstruction with 3D-DOCTOR)

A 3D-DOCTOR supports a variety of 3D file formats. ‘.3ds’ is the most famous and useful format of Autodesk 3Ds Max, and it has merits of short loading time and a small file size. Additionally,  almost all industrial 3D programs support this format.

3DVIA Web and Application (app) solution of DASSAULT SYSTEMES was used for sending and using ‘.3ds’ files on an iPad. It can be used after registering on their Web site and uploading ‘.3ds’ files in personal account is available.

(Fig. 2. 3D models uploaded in account of 3DVIA website)

After uploading, one can handle it with 3DVIA Mobile HD app on an iPad.

This app is a viewer for 3D files, and seeing and touching a 3D models on iPad by fingers are possible. Handling motion includes zooming, dragging, spinning and rotating to make an angle that surgeon requires in the operation.

(Fig. 3. 3D model in 3DVIA app on iPad; mucocele, abdominal arteries, colon, and bony structure)

(Fig. 4. Direct handling of 3D object on iPad; Practical application in operation)

Discussion

One of the difficulties faced with most commercialized medical 3D reconstructing programs is that they do not provide an external export function for 3D objects. In most cases, 3D objects made in a 3D reconstructive program must be edited, and viewed in the same program. It is a tacit rule of the 3D reconstructive programs from one of famous vendor to open sourced like ‘Osirix’ for some reasons. Programs provide only functions that make a 2D still image, or via movie clips of 3D object.

Actually, the Osirix offers one of a 3D exporting option, a ‘QuickTime VR’, however, it does not get any formal support from Apple, and it is not a suitable format for secondary processing on 3D programs. It is only a final outcome for viewing. Currently, there is no way to use a Quicktime VR file on iPad, and the official web page of Quicktime VR on the Apple web site has disappeared.9

3D reconstructive programs are expensive. The cost of 3D-DOCTOR for commercial version (medical use) is 4,800 U.S. dollars (3,650 Euros) per license, and an academic version (not to be used for medical treatment) is 2,400 U.S. dollars (1,827 Euros). Compared to open-sourced program such as Osirix (maximum 599 U.S. dollars for 1 year), 3D-DOCTOR is not that economical, although it seems to be less expensive than the popular 3D program which costs over 15,000 U.S. dollars. In this study, one month limited demo version of 3D-DOCTOR program was used, and some functions such as exporting file size were limited.

Since the 3DIVIA solution has not received an approval from FDA, the medical use is yet limited.

We recommend using an 'INDIVIDUAL account' instead of ‘FREE account’ of 3DVIA web solution to secure medical information of patient because an INDIVIDUAL account is guaranteed with closed and private usage of uploaded 3D models. It requires an additional payment of 9 U.S. dollars per month.

Another technical limitation is that file size has to be smaller than 20 MB and the number of triangles of the 3D object must be in 20,000 in almost all 3D applications. Apple had mentioned these limitations in its technical document, ‘iBook Author: Best practices for using 3D models’ too.10 If an object violates these limitations, the 3DVIA web site will reject uploading or possibly part of the model will disappear or break on an iPad viewer. We note that advanced visual effects (surface glossing, transference, et al) in medical 3D program of PC are not supported in iPad apps.

Safety of iPad in operative room could be a critical point in this trial. There is no evidence that supports safety against infection for long and continuous physical contact.11

To conclude, we remark that

3D medical programs, with time, should support exporting to conventional 3D file formats. Each individual image program vendors will not willing to support to export a 3D reconstructed object to different programs. Also, they may try to control entire process from PC to mobile device with their own expensive 3D solutions.

However, we believe that it can be embodied with easy and cheap way.

Mobile device technology is developing rapidly. For example, the Apple doubled memory capacity of iPad for each model, and improved resolution and 3D processing.

‘COLLADA’ is only one standard 3D file format that Apple approved to iPad and it has '.dae' file extension.12 Currently, there is no 3D medical program that supports '.dae' file format , but with advancement in 3D medical technology, we hope to see a 3D object on iPad directly without using viewers such as jpg pictures.

.

Conclusion

Although, there are many limitations, this study is worth to find a possibility of using mobile devices as end-terminals for Navigation surgery or augmented reality. This field has a great positive potential energy, and we expect vigorous activities of medical staffs in this field.

iPad (1st generation, 2010). Apple Inc. Cupertino, CA 95014, USA. http://www.apple.com/.

Osirix Imaging Software. ⓒPixmeo Sarl. http://www.osirix-viewer.com/.

AutoCAD. Autodesk, Inc. San Rafael, CA 94903, USA. http://usa.autodesk.com/autocad-products/.

Autodesk 3ds Max. Autodesk, Inc. San Rafael, CA 94903, USA. http://usa.autodesk.com/3ds-max/.

3D-DOCTOR (4.0.20110710, 1998-2011, demo). Able Software Corp. Lexington, MA 02420, USA. http://www.ablesw.com/.

3DVIA. DASSUALT SYSYEMES. Velizy-Villacoublay, France. http://www.3dvia.com/.

3DVIA mobile HD (1.4, 2011). DASSUALT SYSYEMES. Velizy-Villacoublay, France. http://itunes.apple.com/us/app/3dvia-mobile-hd/id383844825?mt=8.

The Apple allowed numbers of triangle from 20,000 to 50,000 for the new retina display iPad announced in 2012.

COLLADATM. https://collada.org/mediawiki/index.php/COLLADA_-_Digital_Asset_and_FX_Exchange_Schema.